Vibe-coding flow for C# WLAN EVM Analysis

I had previously implemented a python vibe coding example. The SA side went reasonably well, though Cursor tripped over things like assuming NI RF drivers were VISA. I (and Cursor) struggled with the RFSG/RFSG-PB side, and I eventually resorted to SA only. The focus then moved to C# based on the use-case of a specific customer.

I initially explained that the goal of the application was an optimized WLAN EVM example that would generate a 320MHz WLAN BE waveform and do a modulation accuracy measurement. I indicated that we would start with a loopback CW measurement and build up to the full working measurement step-by-step. I also indicated that we would start with a console app to make sure the RF side was working before going moving to a GUI. Based on these items, I asked Cursor to create a scalable file structure.

I submitted this query to Gemini: “I am looking for the best, most complete C# API documentation available for RFmxInstr, RFmxSpecan and RFmxWLAN. This will help when vibe-coding so the given AI can have the best accuracy”

I posted Gemini’s results to the Cursor chat, then repeated for NI-RFSG and NI-RFSG Playback Library, which has .Net wrappers. This would have been better handled using “Cursor Docs”. This would have enabled all of these documents to be indexed and @mentioned later within the chat.

I also pointed to the named on-disk dotNet examples that install with our RF drivers. Each example folder contains subfolders for “cs” and “vb”. have a “cs” folder and a “vb” folder. This would have been best handled by Cursor Rules where I could have excluded the vb folders and told cursor to reference them as best-practices examples, and to refer to their flow rather than re-inventing one.

I started the coding by requesting these successive tasks:

- Generate a CW tone from the SG (which I viewed in IS)

- Generate a CW tone via a script (Viewed in IS)

- Generate a 320MHz WLAN BE signal via a script (Viewed in IS)

Next was the SA

- Detect a tone and return peak power (Generate from IS)

- Channel power measurement on 320MHz WLAN BE (Generate from IS)

Next steps, starting with a console app:

- Full CW loopback with script

- Generate 320MHz WLAN BE waveform from SG, measure EVM with default settings, return average rms EVM

- Change Channel Estimation type to “Reference and Data” (optimization)

- Add general-purpose auto-level

- Follow-on with OFDM-ModAcc AutoLevel

Once working I asked Cursor to create a windows form:

- Provide user inputs for

  - Center Frequency

  - 802.11 version

  - Waveform path

  - Waveform bandwidth

- Sweep through a set of power levels provided by user input

- Measure EVM at each iteration

- Display the immediate constellation plot for each iteration

- Build up an EVM bathtub curve that updates dynamically with each loop iteration

The looping wasn’t too difficult for Cursor to figure out, and with the Window form, Cursor was in very familiar territory. Cosmetic tweaks to the GUI were accomplished by taking screenshots and explaining why the current organization made things less clear to a user.